where,v=dt= mean streamer velocity (cm/μs)d= electrode gap (cm)t=effective pulse width  (μs)      [use  time >63%  of peak  if not  square]V= peak voltage (MV)units are mixed

| Medium | Enhanced Electrode Polarity | Equations |
| --- | --- | --- |
| Water (V>1MV) | (+) or (-) | vt12=V0.09  t=0.09dV2 |
| Water (V<1MV) | (+) | vt12=8.8V0.6  t=d8.8V0.62 |
|  | (-) | vt13=16V1.09  t=d16V1.0932 |
| Oil | (+) | v=90V1.75  t=d90V1.75 |
|  | (-) | v=31V1.28  t=d31V1.28 |
| Polyethylene | (+) | v=190V1.00  t=d190V1.00 |

v(velocity)

| Medium | Electrode | Equation |
| --- | --- | --- |
| Water | (+) | vt12=8.8V0.6 |
|  | (-) | vt13=16V1.09 |
| Oil | (+) | v=90V1.75 |
|  | (-) | v=31V1.28 |
| Polyethylene | (+) | v=190V1.00 |

`